Taking the biophysical heat model out of a controlled lab and deploying it into a live data pipeline requires a fundamental shift. We must replace ingestible telemetry pills, gravimetric scales, and physical globe thermometers with continuous API webhooks and high-fidelity wearable sensors.

By utilizing edge computing from modern wearables (e.g., Apple Watch Ultra, Oura, Garmin) combined with hyper-local weather APIs, we can estimate physiological strain in real-time. Here is the Wearable-Adapted Thermoregulatory Performance Model (wATPM), designed specifically for automated, remote data collection.

### 1. The Core Environmental Data Streams (API & Wearable)

To run this model dynamically without manual intervention, our data extraction points change to the following:

- API-Derived WBGT ($eWBGT$): Instead of onsite hardware, we ping environmental APIs (e.g., Tomorrow.io, OpenWeatherMap) using the athlete's exact GPS coordinates. The API provides ambient temperature ($T_a$), relative humidity ($RH$), wind speed ($v$), and solar radiation ($W/m^2$). We use an established meteorological algorithm (like the Liljegren method) to calculate the estimated Wet Bulb Globe Temperature remotely.

- Wearable-Estimated Core Temp ($eT_{core}$): Ingestible pills are impractical for daily use. We substitute this with an algorithmic estimation utilizing continuous Heart Rate ($HR$) and Skin Temperature ($T_{skin}$) pulled directly from the athlete's smartwatch or biosensor ring.

- Algorithmic Fluid Loss Estimation ($e\Delta Mass$): Since we cannot weigh the athlete naked in the field constantly, we estimate sweat volume dynamically by cross-referencing their baseline sweat constant with the real-time active energy expenditure (kCal/min) or power output (Watts) tracked by their wearable.

### 2. The Wearable-Adapted Thermoregulatory Performance Model (wATPM)

This model calculates Expected Power Output ($P_{exp}$) using only data streams accessible via REST APIs or wearable SDKs.

$$P_{exp} = 100 - \left[ \left( \omega_1 \cdot e\Phi_{thermal} + \omega_2 \cdot e\Phi_{fluid} + \omega_3 \cdot e\Phi_{metabolic} \right) \times \Gamma_{acclimation} \right]$$

#### 1. The Estimated Thermal Load Vector ($e\Phi_{thermal}$)

This calculates the vapor pressure gradient using API data, determining if the athlete's sweat will actually evaporate in the target environment (e.g., the intense heat of Qatar).

$$e\Phi_{thermal} = \frac{eE_{req}}{eE_{max}}$$

- $eE_{req} \approx (k_{wearable} \cdot HR_{active}) + (API_{Solar} \cdot C_{clothing})$: We estimate metabolic heat production ($M$) using the wearable's continuous active heart rate, adding the solar radiation load pulled from the weather API.

- $eE_{max} \approx v_{API}^{0.6} \cdot (P_{sk} - P_{API\_vapor})$: The maximum evaporative capacity is calculated using the API's wind speed ($v_{API}$) and the gradient between saturated skin vapor pressure ($P_{sk}$, derived from wearable $T_{skin}$) and the ambient vapor pressure ($P_{API\_vapor}$, derived from API temperature and humidity).

#### 2. The Algorithmic Fluid Vector ($e\Phi_{fluid}$)

This calculates the cardiovascular penalty of plasma loss without using a scale.

$$e\Phi_{fluid} = e^{(k \cdot e\%BML)}$$

- $e\%BML = \frac{S_{base} \cdot \int (HR_{active} - HR_{rest}) dt}{Mass_{initial}}$: We estimate the percentage of Body Mass Loss by taking the athlete's known baseline sweat rate ($S_{base}$) and multiplying it by the integral of their active heart rate over time (tracked via smartwatch), divided by their pre-synced body weight in Apple Health/Google Fit.

#### 3. The Wearable Metabolic Shift Vector ($e\Phi_{metabolic}$)

This penalizes output based on how close the brain is to forcing a thermal shutdown, using the estimated core temperature.

$$e\Phi_{metabolic} = \frac{eT_{core(t)} - 37.0}{39.5 - 37.0}$$

- $eT_{core(t)} = T_{skin} + \alpha (HR_{active} - HR_{rest})$: Using the Extended Kalman Filter approach (buller et al.), the wearable's real-time $T_{skin}$ is adjusted by the cardiovascular strain (HR) to accurately predict internal core temperature without a physical probe.

### Part 3: The Automated Dynamic Stabilization Protocol

With data flowing automatically, interventions are triggered by digital alerts rather than manual coaching observations:

- API-Triggered Pre-Cooling ($eE_{req}$ Management):

  - Trigger: A webhook detects that the API forecast for the event location's $eWBGT$ will exceed 26°C.

  - Action: An automated alert dictates the ice slurry ingestion protocol 45 minutes prior to the start time.

- Wearable-Triggered Rehydration ($e\Phi_{fluid}$ Management):

  - Trigger: The smartwatch algorithm calculates that $e\%BML$ has reached the 1.5% threshold based on total accumulated energy expenditure.

  - Action: A haptic alert on the wearable prompts the athlete to consume a specific volume of tailored electrolyte fluid, bypassing the flawed "drink to thirst" mechanism.

- Real-Time Pacing Adjustments ($e\Phi_{metabolic}$ Management):

  - Trigger: The wearable calculates $eT_{core(t)}$ crossing 38.8°C with a rising trajectory.

  - Action: The central governor is approaching its limit; the athlete is signaled to drop their mechanical power output (Watts) by 5-10% to prevent catastrophic heat storage and a DNF (Did Not Finish).

### Updated Peer-Reviewed Literature (Wearable & API Validation)

To validate this automated pipeline, we draw upon research proving the efficacy of non-invasive sensors and meteorological approximations:

- Wearable Core Temperature Estimation

  - Buller, M. J., et al. (2013). Estimation of human core temperature from sequential heart rate observations.(Physiological Measurement).

  - Principle: Validates the algorithm used to predict $T_{core}$ continuously using only heart rate time-series data, proving it is robust enough for field deployment without ingestible pills.

  - PubMed/23996078

- Weather API vs. Onsite Micro-Climates

  - Liljegren, J. C., et al. (2008). An improved approximation for the wet bulb globe temperature (WBGT).(Journal of Occupational and Environmental Hygiene).

  - Principle: Provides the mathematical framework for accurately calculating WBGT using standard meteorological data (temperature, humidity, wind, solar radiation) accessible via modern weather APIs, bypassing the need for physical globe thermometers.

  - PubMed/18720259

- Wearable Energy Expenditure and Sweat Estimation

  - Düking, P., et al. (2016). Comparison of non-invasive individual monitoring of the training and health of athletes with commercially available wearable technologies. (Frontiers in Physiology).

  - Principle: Examines the validity of high-end consumer wearables in accurately tracking energy expenditure and autonomic strain (HRV/HR), which serves as the foundational data stream for calculating $e\%BML$dynamically.

  - PMC4754445

- Skin Temperature as a Thermoregulatory Proxy

  - Welles, A. P., et al. (2017). Continuous skin temperature monitoring to predict heat strain. (Medicine & Science in Sports & Exercise).

  - Principle: Demonstrates how continuous peripheral skin temperature ($T_{skin}$) from wrist-based or ring wearables can be effectively combined with heart rate to flag critical thermal strain before the central governor forces mechanical failure.

  - PubMed/28141618